Sample Numerical on IoT Aspects

Q1) An MQTT message contains:

- Payload = 50 bytes

- Header = 10 bytes

If 1000 messages are sent per hour, calculate total data transmitted in KB.

### Solution

Total per message:

50 + 10 = 60 { bytes}

Per hour:

1000 x 60 = 60000  bytes

Convert to KB:

60000 /1024 = 58.6   KB

Total = 58.6 KB

Q2) An IoT system has:

- Sensor processing delay = 20 ms

- Transmission delay = 50 ms

- Cloud processing = 80 ms

- Response delay = 40 ms

Calculate total system latency.

### Solution

Total = 20 + 50 + 80 + 40

Total latency = 190 ms

Q3) 500 smart meters send 32-byte readings every 15 minutes.

Calculate storage needed for 1 year (in GB).

### Solution:

### Readings per hour:

60/15 = 4

Per day:

4 x 24 = 96

Per year:

96 x 365 = 35040

Per meter per year:

35040 x 32 = 1,121,280 bytes

For 500 meters:

1,121,280 x 500 = 560,640,000  bytes

Convert to GB:

560,640,000 / (1024)3  ≈ 0.52 GB per year

Q4) A LoRa system uses:

- Bandwidth (BW) = 125 kHz

- Spreading Factor (SF) = 10

- Coding Rate (CR) = 4/5

Calculate the approximate data rate.

Solution:

LoRa bit rate formula:

Substitute values:

= 976.5 bps

Data rate ≈ 977 bps

Q5) A LoRa packet contains 20 bytes payload.

SF = 9, BW = 125 kHz.

Calculate symbol duration.

Solution:

Symbol duration:

= 0.004096 sec

Symbol duration = 4.096 ms

Q6) ZigBee operates at 250 kbps.

A sensor sends a 200-byte packet.

Calculate transmission time.

### Solution

Convert to bits:

200 x 8 = 1600  bits

Transmission time:

1600 / 250000 = 0.0064  sec

Transmission time = 6.4 ms

Q7) In a ZigBee tree network:

- Maximum children per parent (Cm) = 5

- Maximum routers (Rm) = 3

- Maximum depth (Lm) = 4

Estimate maximum number of nodes (approximate).

### Solution

Approximate nodes:

Level 0: 1 (Coordinator)

Level 1: 5

Level 2: 52 = 25

Level 3: 53 = 125

Level 4: 54 = 625

Total:

= 1 + 5 + 25 + 125 + 625

Maximum ≈ 781 nodes (approximate theoretical)

Q8) Standard IPv6 header = 40 bytes

After 6LoWPAN compression = 6 bytes

Calculate percentage reduction.

### Solution

Reduction:

40 - 6 = 34  bytes

Percentage:

34 /40 x100

= 85%

Header reduced by 85%

Q9) IEEE 802.15.4 frame size = 127 bytes

MAC header = 25 bytes

Remaining payload capacity?

### Solution

127 - 25 = 102  bytes

If IPv6 packet size = 600 bytes

Number of fragments:

600/102 = 5.88

Round up: 6 fragments required

Q10) A CoAP client sends a 60-byte message over a 250 kbps IEEE 802.15.4 link.

Calculate transmission time.

### Solution:

Convert to bits:

60 x 8 = 480 bits

t =  480 / 250000 = 0.00192  sec = 1.92  ms

Answer: 1.92 ms